Application of LED Moving Sign Controller

Communication Protocol of C32S02

V1.20

1. About Controller System	4

1.1 Communication Port	4

1.2 Features	4

2 Communication Protocol	5

2.1 Communication basic	5

1) Sending Format: Send data to LED Moving Sign	5

2) Return Format: Receive return from LED Moving Sign	5

3) The define of Escape character	6

2.2 Error Return Code	6

3. Program Download Command	7

3.1.1 Command INST: Sending Text Program (OTXT)	7

3.1.2 Command OTXT:  Append Text to (OTXT) Program	8

3.1.3 Command OPGM : Multi Windows Program	9

3.2 Command OGIF: Sending Bitmap/Animation	10

Step 1. Send First Package	11

Step 2. Send Data Package (Multi times sending till end of file)	12

3.3 Command OLIB : Sending Resource File	14

3.4 Command OPGM : Multi Windows Program	14

4. Program Switch Command	14

4.1 Command PLAY: Switch Program and save to Flash Memory	14

1) CMD=0x5A, Sending Play Table	14

2) CMD=0x50, Erase Play Table	15

4.2 Command PRAM: Switch Program	16

1)	Play Mode 1, CMD=0x01 Switch Program by File ID range	16

2)	Play Mode 2, CMD=0x10 Switch Program by Order	17

3) CMD=0x00 Exit from Play Schedule	18

4) CMD=0x04 Return the value of RAM Play Schedule Table	19

4.3 Command HOLD: Display pause and continue	19

1) Holding the display, and display the text attachment	19

2) Holding the display, and do not clear screen	20

4.4 Command EVEN: Switch Program by Event	20

5. Variable Communication Command	21

5.1 Command VROM: Sending Flash Variable	21

5.2 Command VRAM: Sending RAM Variable	22

6 Program Manager Command	23

6.1 Command ALAS: Reading the file name (Alias)	23

1) Overview of Alias	23

2) Find First Program File ID	23

3) Find Next Program File ID	24

4) Return: Program existed and return the Alias	25

5) Return: No more program	25

6) Query Current Running Program Alias	26

6.2 Command DELE: Delete files by File ID Range	26

6.3 Command DELP: Delete all program by File ID range	26

6.4 Command DALL: Delete all files	27

6.5 Command GETX: Return a valid file ID	28

6.6 Command RSET: Reset the display	29

6.7 Command FMAT: Initial the display	29

6.8 Command TIME: Sending Current Time	29

6.9 Command PWON: Software Power On/Off	30

1) Sending Power Schedule	30

2) Get Power Status, and the schedule of Power	31

6.10 Command BRIG: Adjust the brightness (8 scales)	31

6.11 Command QVER: Query the version of firmware	32

6.12 Command GIMG: Return Current Display Image from sign	32

6.13 Command STID: Setup ID & Other functions	34

0) 0x00 Query Sign ID and Sign MAC Address	34

1) 0x01 Setup ID according to MAC address	35

2) 0x11 Enter Sign ID Manual Setup Function	35

3) 0x21 Enter Timer Function	35

4) 0x22 Timer START	36

5) 0x23 Timer PAUSE	36

6) 0x24 Timer STOP/CLEAR	36

7) 0x10 Exit Function of Setup ID or Timer	37

6.14 Command BEEP: Beep Sound	37

7. Feature and Escape String	38

7.1 How to display existed image in text string	38

7.2 Normal Escape String	38

7.3 Color Code and Color Escape String	39

7.4 Action Code	39

7.5 Asian/English Font ID Escape String	40

7.6 Stay Time Code	40

# 1. About Controller System

## 1.1 Communication Port

1.  2 RS232 interface COM1 and COM2, or 1 RS485.

2.  COM1 Normally communication with PC, we define COM1, please kindly noted.

3.  COM2 Normally for user special using need protocol.

## 1.2 Features

- “ What you see as what you get” powerful display, edition effect even for your logo icon.

- Full color text file with flashing, deformation, rainbow, edge, background and animation etc.

- Multi windows and window size and position could be set as you want.

- More than 20 actions as snow, windmill, laser, balloon rising etc.

- Support RTF file and all windows fonts, meanwhile allowing multiple languages in one file.

- Dozens of border patterns (or extended) including the neon water boarder and 4 edges of opening and closing.

- Rich image processing, even logo icon and animation.

- ID group management via embedded software, GPRS,TCPIP modules.

- History file management.

- Countdown and alarm functions, 8 brightness scales and remoter control.

- V3.0 above support USB keyboard input.

- SCM software update without shell opening.

- Hardware could be extendable, suitable for 1/16 duty driver program and support ordering and carousel playing.

- Powerful second development, String File and special scanning mode.

# 2 Communication Protocol

## 2.1 Communication basic

We send the data via a RS232 communication port, and the communication format is baud rate57600，n，8，1

Also you can use another baud rate, please refer the option of Protocol Test.exe

### 1) Sending Format: Send data to LED Moving Sign

| Name | Head | Sign ID | Bytes | Command | Data | CRC | Rear |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bytes | 2 | 2 | 2 | 4 | N | 2 | 2 |
| Demo | 0x7E,0x01 |  |  | “ABCD” |  | CRC | 0x7E,0x00 |

Sign ID ：The ID of LED Moving Sign, 2 bytes

0xfe,0xfe is default ID, it will get Return from LED Sign.

0xff,0xff is Broadcast ID, no return from LED Sign

Bytes:  Package bytes length, include Command and the length of Data. 2 bytes, and first byte is high byte of word

CRC check: We can use 0xff,0xff to pass the CRC check.

### 2) Return Format: Receive return from LED Moving Sign

| Name | Head | Sign ID | Bytes | Cmd | Ret | Length Bytes | Return Code | CRC | Rear |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bytes | 2 | 2 | 2 | 4 | 1 | 1 | N | 2 | 2 |
| Demo | 0x7E,0x02 |  |  | “ABCD” | 0x00 |  |  | CRC | 0x7E,0x00 |

Notice: Return Code == 0x00, Communication is OK, and other value means there’s error occur.

### 3) The define of Escape character

The escape character is 0x7e, only existed in frame head, and frame rear, it must be convert to 2 bytes if there’s 0x7e or 0x7d in data list,

Escape protocol：

（1）Sending:

The 0x7d and 0x7e in data have to convert to 2 bytes code like this

0x7e => 0x7d,0x02

0x7d => 0x7d,0x01

The length of sending still not change (e.g.  0x7d, 0x02, 0x35 = 2 bytes)

（2）Receiving:

0x7d,0x02 => 0x7e

0x7d,0x01 => 0x7d

## 2.2 Error Return Code

0x00  Normal

0xAA  General Error

0x01 Invalid Command

0x02 Length error

0x05 Flash error

0x07 Not empty file

0x08 Can not allow memory

0x09 Invalid file name

0x0A No such file

# 3. Program Download Command

## 3.1.1 Command INST: Sending Text Program (OTXT)

The command INST is used to send text program to LED Moving Sign, LED Controller will new create 3 files ( OPGM file, ORUN file, OTXT file) in sign under the config of INST command.

e.g.

Send a text string “12345” to LED Sign, The Program ID is set to 1,

Sending:

7E 01 FE FE 00 34 49 4E 53 54 00 00 01 00 00 00 00 00 00 00 00 01 00 00 00 00 00 00 00 0F 01 00 05 01 00 03 00 00 01 00 00 00 0F 31 32 33 34 35 36 20 CF AA B4 BA C2 B7 BF DA FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head 7E01/7E43 FE FE ¦&0002 (2)¦ Sign ID 00 34 ¦&0004 (2)¦ Length 49 4E 53 54 ¦&0006 (4)¦ Command 00 ¦&000A (1)¦ Sub Command 00 1E ¦&000B (2)¦ Program ID 00 ¦&000D (1)¦ Windows # 00 ¦&000E (1)¦ Message # 00 ¦&000F (1)¦ Reserve 00 00 ¦&0010 (2)¦ Start Minute 00 00 ¦&0012 (2)¦ End Minute 00 00 ¦&0014 (2)¦ Play Time 00 ¦&0016 (1)¦ Play Times 00 ¦&0017 (1)¦ Alias Length 00 ¦&0018 (1)¦ Reserve 00 ¦&0019 (1)¦ d:Type 00 ¦&001A (1)¦ d:Color 00 0F ¦&001B (2)¦ d:Font(Chn) 00 0F ¦&001D (2)¦ d:Font(English) 01 00 ¦&001F (2)¦ d:nc 05 ¦&0021 (1)¦ d:Stay Time 01 ¦&0022 (1)¦ d:Action 03 ¦&0023 (1)¦ d:Hold Action 00 ¦&0024 (1)¦ d:Speed 00 ¦&0025 (1)¦ d:Roll Pixels 01 ¦&0026 (1)¦ d:Roll Stay 00 ¦&0027 (1)¦ d:Loops 00 ¦&0028 (1)¦ d:Align 00 0F ¦&0029 (2)¦ d:Bytes B9 E3 B8 E6 BD DA C4 BF 41 42 43 44 45 46 47 ¦&002B (15)¦ x:Data FF FF ¦&003A (2)¦ CRC 7E 00 ¦&003C (2)¦ Frame Reer | “INST” 0: Normal, 1:Hiding 3 File ID will be create 0: Default, >0: Windows ID 0: Default, >0: Message ID Not use Not use Seconds <=16, Alias is the head of data Display or not Text is 0 , always 0 0 Auto,1 Red,2 Green,3 Yellow... Set to same of English Seconds 0, CH4/CL4，F1,F2,F3,F4=FlashMode * 0 Default, >1 Speeds Not use Seconds Not use Not use 15 Bytes Alias + Text 123456xxx… |

Return code:

7E 02 00 01 00 06 49 4E 53 54 00 00 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Frame Head 00 01 ¦&0002 (2)¦ Sign ID 00 06 ¦&0004 (2)¦ Data Length 49 4E 53 54 ¦&0006 (4)¦ Command 00 ¦&000A (1)¦ Return Flag 00 ¦&000B (1)¦ Reserve FF FF ¦&000C (2)¦ CRC 7E 00 ¦&000E (2)¦ Frame Reer | INST |

## 3.1.2 Command OTXT:  Append Text to (OTXT) Program

Command OTXT, Append text to existed program. (OWND Files)

| Command OTXT: Send string “12345” to ID1 | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ 帧头7E01/7E43 FE FE ¦&0002 (2)¦ 屏ID/长度 00 34 ¦&0004 (2)¦ 长度/流水ID 4F 54 58 54 ¦&0006 (4)¦ 协议命令OTXT 00 ¦&000A (1)¦ New0,Append1 00 1E ¦&000B (2)¦ 节目ID 01 ¦&000D (1)¦ 窗口0-7 00 ¦&000E (1)¦ Reverse 00 ¦&000F (1)¦ Reverse 00 00 ¦&0010 (2)¦ Reverse 00 00 ¦&0012 (2)¦ Reverse 00 00 ¦&0014 (2)¦ Reverse 00 ¦&0016 (1)¦ Reverse 04 ¦&0017 (1)¦ Reverse 01 ¦&0018 (1)¦ 显示项(长d+x) 00 ¦&0019 (1)¦ d:类型:文字0 00 ¦&001A (1)¦ d:色彩:0X1R2G3Y 02 10 ¦&001B (2)¦ d:字体(中) 01 08 ¦&001D (2)¦ d:字体(英) 01 00 ¦&001F (2)¦ d:nc 05 ¦&0021 (1)¦ d:停留时间 02 ¦&0022 (1)¦ d:进入动作 00 ¦&0023 (1)¦ d:保持动作 00 ¦&0024 (1)¦ d:速度 00 ¦&0025 (1)¦ d:滚动间距 00 ¦&0026 (1)¦ d:滚动停留 00 ¦&0027 (1)¦ d:循环次数 00 ¦&0028 (1)¦ d:对齐方式 00 0F ¦&0029 (2)¦ d:BYTES 31 32 33 34 BD DA C4 BF 41 42 43 44 45 46 47 ¦&002B (15)¦ x:内容 FF FF ¦&003A (2)¦ CRC 7E 00 ¦&003C (2)¦ 帧尾 | “OTXT” 发送节目指令 00 New,01Append Porgam ID 0: Default Windows 是否有属性内容0为有，1无 未启用 0X,1R,2G,3Y... yes yes 1,2,3,4=闪动 yes 未启用 yes 未启用 未启用 15字节 内容 123456xxx… |

Return :

7E 02 00 01 00 06 xxxx 49 4E 53 54 00 01 FF FF 7E 00

| Return: | 说明: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ 指令帧头 00 01 ¦&0002 (2)¦ 屏ID号 00 06 ¦&0004 (2)¦ 数据长度 4F 54 58 54 ¦&0006 (4)¦ 协议命令 00 ¦&000A (1)¦ 返回标志 01 ¦&000B (1)¦ 返回指示 FF FF ¦&000C (2)¦ CRC 7E 00 ¦&000E (2)¦ 帧尾 | OTXT 指示追加的内容 |

## 3.1.3 Command OPGM : Multi Windows Program

INST Command have the ability to create windows in LED sign. (Left/Right) (Up/Down)

Command:

| Command: OPGM | 说明: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ 指令帧头 FE FE ¦&0002 (2)¦ 屏ID号 00 38 ¦&0004 (2)¦ 数据长度 4F 50 47 4D ¦&0006 (4)¦ 协议命令 00 1E ¦&000A (2)¦ 文件ID(10x) 00 ¦&000C (1)¦ 子命令0为窗口 02 ¦&000D (1)¦ 窗口数 00 00 20 00 00 00 00 00 00 00 00 00 00 00 00 00 ¦&000E (16)¦ d:窗口X 20 00 20 00 00 00 00 00 00 00 00 00 00 00 00 00 ¦&001E (16)¦ d:窗口W 00 00 00 00 00 00 00 00 ¦&002E (8)¦ d:窗口Y 20 20 00 00 00 00 00 00 ¦&0036 (8)¦ d:窗口H FF FF ¦&003E (2)¦ CRC 7E 00 ¦&0040 (2)¦ 帧尾 | “OPGM” 发送节目指令 0为窗口划分 必须不为0 窗X偏移32(0x20) 点表示为:20 00 最大8分区 窗宽32(0x20) 点表示为:20 00 最大8分区 单字节表示窗垂直位置 单字节表示窗高 |

| Return: |  |
| --- | --- |

## 3.2 Command OGIF: Sending Bitmap/Animation

- OGIF Command can send our not only OGIF file (Animation), but also OBMP file (Picture).

- The Program ID need set while use OGIF Command, There’re 3 files will be created automatically in LED Moving Sign.

- OGIF command can be use to send a big file, but we need to cut the file to many pages while sending. The max 248 bytes is allowed in a page.

- We use page SN to control the progress of page communication.

- The progress of a real OGIF sending, such as the the length of OBMP file have to cut to 6 pages, the communication like this,

  - First Sending,Send Initial data

  - 2nd Sending, Send data of Page 0, LED Sign save to page0, return next page ID of 1

  - 3rd Sending, Send data of Page 1, LED Sign save to page1, return next page ID of 2

  - 4th Sending, Send data of Page 2, LED Sign save to page2, return next page ID of 3

  - 5th Sending, Send data of Page 3, LED Sign save to page3, return next page ID of 4

  - 6th Sending, Send data of Page 4, LED Sign save to page4, return next page ID of 5

  - 7th Sending,Send data of Page 5, LED Sign save to page5, return end code of 0xffff

### Step 1. Send First Package

Sending: Initial OGIF/OBMP Communication

7E 01 FE FE 00 38 4F 47 49 46 00 C8 FE FE 00 15 86 20 55 00 00 00 00 00 00 00 00 00 00 4D 6F 76 69 65 5F 30 34 2E 6F 67 69 66 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head FE FE ¦&0002 (2)¦ Sign ID 00 38 ¦&0004 (2)¦ Bytes 4F 47 49 46 ¦&0006 (4)¦ Cmd OGIF 00 C8 ¦&000A (2)¦ Program ID FE FE ¦&000C (2)¦ Package No. 00 15 ¦&000E (2)¦ Total Packages 86 ¦&0010 (1)¦ OGIF/ OBMP 20 ¦&0011 (1)¦ Length <=248 55 ¦&0012 (1)¦ 0x55 00 ¦&0013 (1)¦ 0 New,1 Hide 00 ¦&0014 (1)¦ x 00 ¦&0015 (1)¦ x 00 00 ¦&0016 (2)¦ Pgm Start Minute 00 00 ¦&0018 (2)¦ Pgm Stop Minute 00 00 ¦&001A (2)¦ Pgm Play Seconds 00 ¦&001C (1)¦ Pgm Play Times 4D 6F 76 69 65 5F 30 34 2E 6F 67 69 66 00 00 00 ¦&001D (16)¦ Pgm Alias 00 ¦&002D (1)¦ x 00 ¦&002E (1)¦ d:x 00 ¦&002F (1)¦ d:Color:0X1R2G3Y 00 00 ¦&0030 (2)¦ d:Font Asian 00 00 ¦&0032 (2)¦ d:Font English 00 00 ¦&0034 (2)¦ d: 00 ¦&0036 (1)¦ d:Stay Seconds 00 ¦&0037 (1)¦ d:Action 00 ¦&0038 (1)¦ d: 00 ¦&0039 (1)¦ d:Speed 00 ¦&003A (1)¦ d: 00 ¦&003B (1)¦ d: 00 ¦&003C (1)¦ reserve 00 ¦&003D (1)¦ d:x Align FF FF ¦&003E (2)¦ CRC 7E 00 ¦&0040 (2)¦ Frame Rear | 0x28 – 0x38 “OGIF” Command Program ID 0xfefe is Initial page Total Package Number File Prop. OGIF为0X86 **** No use No use Time Time Time Time |
| Return : 7E 02 00 01 00 0C 4F 47 49 46 00 00 00 20 00 01 00 11 FF FF 7E 00 | Description |
| 7E 02 ¦&0016 (2)¦ Head 00 01 ¦&0018 (2)¦ Sign ID 00 0C ¦&001A (2)¦ Length 4F 47 49 46 ¦&001C (4)¦ Command OGIF 00 ¦&0020 (1)¦ Answer 00 ¦&0021 (1)¦ 00 20 ¦&0022 (2)¦ Program ID 00 01 ¦&0024 (2)¦ SN 00 11 ¦&0026 (2)¦ Write Bytes FF FF ¦&0028 (2)¦ CRC 7E 00 ¦&002A (2)¦ Rear | “OGIF” 00 is normal ,AA is error |

### Step 2. Send Data Package (Multi times sending till end of file)

7E 01 FE FE 01 04 4F 47 49 46 00 C8 00 00 00 14 86 F8 86 00 47 00 A6 00 12 02 35 00 00 00 05 00 00 00 CC 00 02 02 01 00 00 00 00 00 00 00 A6 00 00 12 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 37 35 35 37 35 35 36 35 B5 34 35 F5 35 35 F5 35 35 55 35 35 05 35 35 0D 35 35 0D 35 35 19 35 35 13 35 35 16 35 35 14 35 B5 15 35 F5 15 35 D5 15 35 95 15 35 A5 15 35 AD 15 35 BD 15 35 BD 15 35 BD 15 35 BD 35 35 BD 35 35 BD 35 35 BD 35 35 FF FF 7E 00

| Command : Send Packages | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head FE FE ¦&0002 (2)¦ Sign ID 01 04 ¦&0004 (2)¦ Length of data 4F 47 49 46 ¦&0006 (4)¦ Command OGIF 00 C8 ¦&000A (2)¦ Program ID 00 00 ¦&000C (2)¦ Package No. 00 14 ¦&000E (2)¦ Total Package Num. 86 ¦&0010 (1)¦ OGIF/OBMP File F8 ¦&0011 (1)¦ Length<=248 86 00 47 00 A6 00 12 02 35 00 00 00 05 00 00 00 CC 00 02 02 01 00 00 00 00 00 00 00 A6 00 00 12 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 35 37 35 35 37 35 35 36 35 B5 34 35 F5 35 35 F5 35 35 55 35 35 05 35 35 0D 35 35 0D 35 35 19 35 35 13 35 35 16 35 35 14 35 B5 15 35 F5 15 35 D5 15 35 95 15 35 A5 15 35 AD 15 35 BD 15 35 BD 15 35 BD 15 35 BD 35 35 BD 35 35 BD 35 35 BD 35 35 ¦&0012(248)¦ Data of R/G/B FF FF ¦&010A (2)¦ CRC 7E 00 ¦&010C (2)¦ Rear | 0xFeFe is manager package |
| Return: | Description: |
| 7E 02 ¦&0016 (2)¦ Frame Head 00 01 ¦&0018 (2)¦ Sign ID 00 0C ¦&001A (2)¦ Length 4F 47 49 46 ¦&001C (4)¦ Command OGIF 00 ¦&0020 (1)¦ Answer 00 ¦&0021 (1)¦ 00 20 ¦&0022 (2)¦ Program ID 00 01 ¦&0024 (2)¦ SN 00 11 ¦&0026 (2)¦ Write Bytes FF FF ¦&0028 (2)¦ CRC 7E 00 ¦&002A (2)¦ Rear | OGIF |

## 3.3 Command OLIB : Sending Resource File

Will be come…

## 3.4 Command OPGM : Multi Windows Program

Will be come…

# 4. Program Switch Command

## 4.1 Command PLAY: Switch Program and save to Flash Memory

The command PLAY is used to set the Program Play Schedule, which is saved in Flash Memory.

The Flash play schedule will start when LED Sign power on.

### 1) CMD=0x5A, Sending Play Table

e.g.

Sending play schedule to LED Sign, set running program group ‘A’, all programs in LED Sign. And current display program will be interrupted immediatly after communication, and LED Sign will running the new play schedule.

Sending:

7E 01 FE FE 00 0E 50 4C 41 59 5A 01 00 06 41 01 00 01 00 01 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head,7E01 FE FE ¦&0002 (2)¦ Sign ID 00 0E ¦&0004 (2)¦ Cmd Length 50 4C 41 59 ¦&0006 (4)¦ Command 5A ¦&000A (1)¦ Sub Command 01 ¦&000B (1)¦ Action right now 00 06 ¦&000C (2)¦ Data Length 41 ¦&000E (1)¦ Group A-Z 01 ¦&000F (1)¦ Data Package Num 00 01 ¦&0010 (2)¦ Start File ID 00 01 ¦&0012 (2)¦ End File ID FF FF ¦&0014 (2)¦ CRC 7E 00 ¦&0016 (2)¦ Frame Reer7E,00 | “PLAY” 0x50 Clear/ 0x5A Set 1 right now, 0 wait till end of program ‘A’-‘Z’ 1 000 999 |

Return code:

7E 02 00 01 00 06 50 4C 41 59 00 5A FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Frame Head 00 01 ¦&0002 (2)¦ Sign ID 00 06 ¦&0004 (2)¦ Data Length 50 4C 41 59 ¦&0006 (4)¦ Command 00 ¦&000A (1)¦ Return Flag 5A ¦&000B (1)¦ Reserve FF FF ¦&000C (2)¦ CRC 7E 00 ¦&000E (2)¦ Frame Reer | 00 is OK, >0 Error |

### 2) CMD=0x50, Erase Play Table

Sending:

7E 01 FE FE 00 0E 50 4C 41 59 50 01 00 06 41 01 00 01 00 01 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&0018 (2)¦ Frame Head,7E01 FE FE ¦&001A (2)¦ Sign ID 00 0E ¦&001C (2)¦ Cmd Length 50 4C 41 59 ¦&001E (4)¦ Command 50 ¦&0022 (1)¦ Sub Command 01 ¦&0023 (1)¦ Action right now 00 06 ¦&0024 (2)¦ Data Length 41 ¦&0026 (1)¦ GroupA-Z 01 ¦&0027 (1)¦ Data Package Num 00 01 ¦&0028 (2)¦ Start File ID 00 01 ¦&002A (2)¦ End File ID FF FF ¦&002C (2)¦ CRC 7E 00 ¦&002E (2)¦ Frame Reer7E,00 | “PLAY” 0x50 Erase/ 0x5A Set play schedule 1 right now, 0 wait till end of program ‘A’-‘Z’ 1 000 999 |

Return code:

7E 02 00 01 00 06 50 4C 41 59 00 50 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Frame Head 00 01 ¦&0002 (2)¦ Sign ID 00 06 ¦&0004 (2)¦ Data Length 50 4C 41 59 ¦&0006 (4)¦ Command 00 ¦&000A (1)¦ Return Flag 50 ¦&000B (1)¦ Reserve FF FF ¦&000C (2)¦ CRC 7E 00 ¦&000E (2)¦ Frame Reer | 00 is OK, >0 Error |

Notice: The Command PLAY enable to run separate program in LED Sign.

## 4.2 Command PRAM: Switch Program

The Command PRAM also switch to new play schedule, but this times the schedule is Ram Play Schedule, stored in RAM memory. It’s dismiss when LED Sign’s power off.

There’re 2 different config modes of RAM Play Schedule,

### Play Mode 1, CMD=0x01 Switch Program by File ID range

All programs in File ID range will be run. e.g. There’re 3 programs in LED Sign, the File ID is 7, 123, 255, and you can send the Program run range 100-300, and the program 123,255 will be run

Also you can setupthe run group to run all program with same group

Sending:  Play programs between File ID 10 to File ID 100

7E 01 FE FE 00 0F 50 52 41 4D 01 01 00 00 01 00 01 41 00 00 00 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head FE FE ¦&0002 (2)¦ Sign ID 00 0F ¦&0004 (2)¦ Data Length 50 52 41 4D ¦&0006 (4)¦ CommandPRAM 01 ¦&000A (1)¦ Sub Command=1/0 01 ¦&000B (1)¦ Action right now 00 ¦&000C (1)¦ Reserve 00 01 ¦&000D (2)¦ Start File ID 00 01 ¦&000F (2)¦ End File ID 41 ¦&0011 (1)¦ GroupA-Z 00 ¦&0012 (1)¦ Loops Times 00 00 ¦&0013 (2)¦ Seconds FF FF ¦&0015 (2)¦ CRC 7E 00 ¦&0017 (2)¦ Frame Reer | “PRAM” 0x01 Set 1 right now, 0 wait till end of program File ID start 1 File ID end 1 ‘A’-‘Z’ Running Seconds |

Return code:

7E 02 00 01 00 07 50 52 41 4D 00 01 00 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Frame Head 00 01 ¦&0002 (2)¦ Sign ID 00 07 ¦&0004 (2)¦ Data Length 50 52 41 4D ¦&0006 (4)¦ Command 00 ¦&000A (1)¦ Return Flag 01 ¦&000B (1)¦ Return Command 00 ¦&000C (1)¦ Data Length0/8 FF FF ¦&000D (2)¦ CRC 7E 00 ¦&000F (2)¦ Frame Reer | 00 is OK, >0 Error CMD = 0x01 |

### Play Mode 2, CMD=0x10 Switch Program by Order

Run programs by Order of Program in Flash Memory. (or range of order, such as 1-3)

e.g. There’re 3 programs in LED Sign, the File ID is 7, 123, 255, and the Order of Program is 1, 2, 3..

you can send the Order of Program to switch them.  (Notice, the order number is below the #9)

Sending: Run the Program with order 1 (Flash Save Position)

7E 01 FE FE 00 0F 50 52 41 4D 10 01 00 00 01 00 01 00 00 00 00 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head FE FE ¦&0002 (2)¦ Sign ID 00 0F ¦&0004 (2)¦ Data Length 50 52 41 4D ¦&0006 (4)¦ CommandPRAM 10 ¦&000A (1)¦ Sub Command:0x10/0 01 ¦&000B (1)¦ Action right now 00 ¦&000C (1)¦ Reserve 00 01 ¦&000D (2)¦ Start Order 00 01 ¦&000F (2)¦ End Order 00 ¦&0011 (1)¦ Reserve 00 ¦&0012 (1)¦ Loops Times 00 00 ¦&0013 (2)¦ Seconds FF FF ¦&0015 (2)¦ CRC 7E 00 ¦&0017 (2)¦ Frame Reer | “PRAM” 0x10 Set Pgm Order/ 0 Clear 1 right now, 0 wait till end of program Program Order 1-9 Program Order 1-9 Running Seconds |

Return code:

7E 02 00 01 00 09 50 52 41 4D 00 10 02 01 01 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Frame Head 00 01 ¦&0002 (2)¦ Sign ID 00 09 ¦&0004 (2)¦ Data Length 50 52 41 4D ¦&0006 (4)¦ Command 00 ¦&000A (1)¦ Return Flag 10 ¦&000B (1)¦ Sub Command 02 ¦&000C (1)¦ Data Length2/8 01 01 ¦&000D (2)¦ Data FF FF ¦&000F (2)¦ CRC 7E 00 ¦&0011 (2)¦ Frame Reer | 00 is OK, >0 Error CMD 0x10, return value Return Data |

### 3) CMD=0x00 Exit from Play Schedule

Sending:

7E 01 FE FE 00 0F 50 52 41 4D 00 01 00 00 01 00 01 41 00 00 00 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head FE FE ¦&0002 (2)¦ Sign ID 00 0F ¦&0004 (2)¦ Data Length 50 52 41 4D ¦&0006 (4)¦ CommandPRAM 00 ¦&000A (1)¦ Sub Command 01 ¦&000B (1)¦ Action right now 00 ¦&000C (1)¦ Reserve 00 00 ¦&000D (2)¦ Start File ID 00 00 ¦&000F (2)¦ End File ID 00 ¦&0011 (1)¦ Group A-Z 00 ¦&0012 (1)¦ Loops times 00 00 ¦&0013 (2)¦ Seconds FF FF ¦&0015 (2)¦ CRC 7E 00 ¦&0017 (2)¦ Frame Reer | “PRAM” 0 clear RAM schedule 1 right now, 0 wait till end of program Mode Running Seconds |

Return code:

7E 02 00 01 00 07 50 52 41 4D 00 00 00 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Frame Head 00 01 ¦&0002 (2)¦ Sign ID 00 07 ¦&0004 (2)¦ Data Length 50 52 41 4D ¦&0006 (4)¦ Command 00 ¦&000A (1)¦ Return Flag 00 ¦&000B (1)¦ Command回传 00 ¦&000C (1)¦ Data Length0/8 FF FF ¦&000D (2)¦ CRC 7E 00 ¦&000F (2)¦ Frame Reer | 00 is OK, >0 Error CMD 00 |

### 4) CMD=0x04 Return the value of RAM Play Schedule Table

This command will return the value of RAM Play Schedule.

## 4.3 Command HOLD: Display pause and continue

### 1) Holding the display, and display the text attachment

Sending: Display “HELLO”, after stop current running program

7E 01 FE FE 00 0B 48 4F 4C 44 01 05 48 65 6C 6C 6F FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&00000 (2)¦ Frame Head FE FE ¦&00002 (2)¦ Sign ID 00 0B ¦&00004 (2)¦ Data Length 48 4F 4C 44 ¦&00006 (4)¦ Command 01 ¦&0000A (1)¦ Sub Command 05 ¦&0000B (1)¦ Bytes 48 65 6C 6C 6F ¦&0000C (5)¦ Text Data FF FF ¦&00011 (2)¦ CRC 7E 00 ¦&00013 (2)¦ Frame Reer | “HOLD” 00 Exit, 01 Hold >0 “HELLO” Display Holding String |

Return Code:

| Code: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Frame Head 00 01 ¦&0002 (2)¦ Sign ID 00 06 ¦&0004 (2)¦ Data Length 48 4F 4C 44 ¦&0006 (4)¦ Command 00 ¦&000A (1)¦ Return Flag FE ¦&000B (1)¦ Control Bit FF FF ¦&000C (2)¦ CRC 7E 00 ¦&000E (2)¦ Frame Reer | >0 / 0xfe |

### 2) Holding the display, and do not clear screen

Sending: Holding the display image of LED Sign after stop current running program.

7E 01 FE FE 00 08 48 4F 4C 44 01 00 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&00000 (2)¦ Frame Head FE FE ¦&00002 (2)¦ Sign ID 00 06 ¦&00004 (2)¦ Data Length 48 4F 4C 44 ¦&00006 (4)¦ Command 01 ¦&0000A (1)¦ Sub Command 00 ¦&0000B (1)¦ Text Bytes FF FF ¦&0000C (2)¦ CRC 7E 00 ¦&0000E (2)¦ Frame Reer | “HOLD” 00 Exit, 01 Hold Display Holding String |

Return Code:

| Code: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Frame Head 00 01 ¦&0002 (2)¦ Sign ID 00 06 ¦&0004 (2)¦ Data Length 48 4F 4C 44 ¦&0006 (4)¦ Command 00 ¦&000A (1)¦ Return Flag FE ¦&000B (1)¦ 控制位FE,00 FF FF ¦&000C (2)¦ CRC 7E 00 ¦&000E (2)¦ Frame Reer |  |

## 4.4 Command EVEN: Switch Program by Event

The priority level of Program Switch is EVEN > PRAM > PLAY,  so if you like to launch a new program when LED Sign is under the PRAM status (Play Ram Schedule is running), you have to use EVEN command, and you can return to PRAM status by EVEN command too.

Usualy, the EVEN command is used to switch Turn-Left, Turn-Right, Stop program in car by signal.

Sending: Run the program 10 by EVEN command.

7E 01 FE FE 00 07 45 56 45 4E 01 00 0A FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head FE FE ¦&0002 (2)¦ Sign ID 00 07 ¦&0004 (2)¦ Data Length 45 56 45 4E ¦&0006 (4)¦ Command 01 ¦&000A (1)¦ Sub Command 00 0A ¦&000B (2)¦ Program ID FF FF ¦&000D (2)¦ CRC 7E 00 ¦&000F (2)¦ Frame Reer | “EVEN” 0 Exit,1 Set Event 00-100 >1023 如FFFF指全部 |

Return Code:

| Code: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Frame Head 00 01 ¦&0002 (2)¦ Sign ID 00 06 ¦&0004 (2)¦ Data Length 45 56 45 4E ¦&0006 (4)¦ CommandEVEN 00 ¦&000A (1)¦ Return Flag 55 ¦&000B (1)¦ Event Flag 55 FF FF ¦&000C (2)¦ CRC 7E 00 ¦&000E (2)¦ Frame Reer | 00 is OK 55 Event Used, 00 Not Use |

# 5. Variable Communication Command

## 5.1 Command VROM: Sending Flash Variable

Max length of Flash Variable is 32 bytes, and there’re 128 Flash Variable in LED Sign.

Notice:

1). Value 0xff is escape code of image position save in resource lib file in LED Sign.

e.g   String “12?34”  is  = 0x31, 0x32, 0xff, 0x01, 0x33, 0x34…

? Means the image position of resource lib file.

2). Value between 0x10 to 0x1f, and not head by 0xff, is color value, e.g. 0x11 is RED, and 0x12 is Green…

e.g.   String “1234” is = 0x31, 0x32, 0x11, 0x33, 0x34

“34” is Red, and “12” is default color.

Sending: “Hello” to Flash Variable #2

7E 01 FE FE 00 0B 56 52 4F 4D 02 05 48 65 6C 6C 6F FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&00000 (2)¦ Frame Head FE FE ¦&00002 (2)¦ Sign ID 00 0B ¦&00004 (2)¦ Data Length 56 52 4F 4D ¦&00006 (4)¦ Command 02 ¦&0000A (1)¦ Variable Name 05 ¦&0000B (1)¦ Variable Length 48 65 6C 6C 6F ¦&0000C (5)¦ Data FF FF ¦&00011 (2)¦ CRC 7E 00 ¦&00013 (2)¦ Frame Reer | “VROM” Max 64 Bytes |

Return code:

7E 02 00 01 00 06 56 52 4F 4D 00 00 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Frame Head 00 01 ¦&0002 (2)¦ Sign ID 00 06 ¦&0004 (2)¦ Data Length 56 52 4F 4D ¦&0006 (4)¦ Command 00 ¦&000A (1)¦ Return Flag 00 ¦&000B (1)¦ Reserve FF FF ¦&000C (2)¦ CRC 7E 00 ¦&000E (2)¦ Frame Reer |  |

Please refer the program of “protocol_text.exe”

## 5.2 Command VRAM: Sending RAM Variable

Max length of RAM Variable is 16 bytes, and there’re 8 RAM Variable in LED Sign

Notice:

1). Value 0xff is escape code of image position save in resource lib file in LED Sign.

e.g   String “12?34”  is  = 0x31, 0x32, 0xff, 0x01, 0x33, 0x34…

? Means the image position of resource lib file.

2). Value between 0x10 to 0x1f, and not head by 0xff, is color value, e.g. 0x11 is RED, and 0x12 is Green…

e.g.   String “1234” is = 0x31, 0x32, 0x11, 0x33, 0x34

“34” is Red, and “12” is default color.

Sending: “Hello” to RAM Variable #2

7E 01 FE FE 00 0B 56 52 41 4D 02 05 48 65 6C 6C 6F FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&00000 (2)¦ Frame Head FE FE ¦&00002 (2)¦ Sign ID 00 0B ¦&00004 (2)¦ Data Length 56 52 41 4D ¦&00006 (4)¦ Command 02 ¦&0000A (1)¦ Variable Name 05 ¦&0000B (1)¦ Variable Length 48 65 6C 6C 6F ¦&0000C (5)¦ Data FF FF ¦&00011 (2)¦ CRC 7E 00 ¦&00013 (2)¦ Frame Reer | “VRAM” 0-7, 0xff is all <=16, or not limited while name=0xff Max 16 bytes |

Return code:

7E 02 00 01 00 06 56 52 41 4D 00 00 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Frame Head 00 01 ¦&0002 (2)¦ Sign ID 00 06 ¦&0004 (2)¦ Data Length 56 52 41 4D ¦&0006 (4)¦ Command 00 ¦&000A (1)¦ Return Flag 00 ¦&000B (1)¦ Reserve FF FF ¦&000C (2)¦ CRC 7E 00 ¦&000E (2)¦ Frame Reer |  |

# 6 Program Manager Command

## 6.1 Command ALAS: Reading the file name (Alias)

### 1) Overview of Alias

Only 1 alias of program be returned by every ALAS communication. So if you like to get all Alias in LED Sign, you have to use ALAS command many times till end of program.

Notice 1: About the Value of Start.

If the value of Start = 0 , it’ll search program from File ID 0, it’s the start of a ALAS communication and the position of program will be saved

If the value of Start > 1023, e.g. 0xffff,  it’ll search program from the saved position of last time.

If the value of Start between 1 to 1023,  it’ll search program from the value of Start.

Notice 2: About the Value of End

If Value of End < Start, then set value of End equal to value of Start

If Value of End > 1023, It means all file range. ( Usually use 0xffff)

If Value of End between 1 to 1023, It’s the end of search range

### 2) Find First Program File ID

Sending:

7E 01 FE FE 00 08 41 4C 41 53 00 00 FF FF FF FF 7E 00

| Code:1. | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head,7E01 FE FE ¦&0002 (2)¦ Sign ID 00 08 ¦&0004 (2)¦ Cmd Length 41 4C 41 53 ¦&0006 (4)¦ Command 00 ¦&000A (1)¦ Sub Command 00 FF FF ¦&000B (3)¦ Reserve,00,FF,FF FF FF ¦&000E (2)¦ CRC 7E 00 ¦&0010 (2)¦ Frame Reer7E,00 | “ALAS” 0x00 0x00ffff for old version |

### 3) Find Next Program File ID

Sending:

7E 01 FE FE 00 08 41 4C 41 53 10 00 FF FF FF FF 7E 00

| Code:2. | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head,7E01 FE FE ¦&0002 (2)¦ Sign ID 00 08 ¦&0004 (2)¦ Cmd Length 41 4C 41 53 ¦&0006 (4)¦ Command 10 ¦&000A (1)¦ Sub Command: 00 FF FF ¦&000B (3)¦ Reserve,00,FF,FF FF FF ¦&000E (2)¦ CRC 7E 00 ¦&0010 (2)¦ Frame Reer7E,00 | “ALAS” 0x10 netxt 0x00ffff for old version |

A new mode after V393, you can use only this command to replace the old 2 commands, but you have to fill the start ID number of next program.

Sending:

7E 01 FE FE 00 08 41 4C 41 53 10 55 01 00 FF FF 7E 00

Notice:

| Code: | 说明: |
| --- | --- |
| 7E 01 ¦&0012 (2)¦ HEAD,7E01 FE FE ¦&0014 (2)¦ SIGN ID 00 08 ¦&0016 (2)¦ CMD LENGTH 41 4C 41 53 ¦&0018 (4)¦ CMD ALAS 10 ¦&001C (1)¦ CMD:0x10 55 ¦&001D (1)¦ NEXT ID NUMBER,0x55 01 00 ¦&001E (2)¦ NEXT ID FF FF ¦&0020 (2)¦ CRC 7E 00 ¦&0022 (2)¦ REAR 7E,00 | ALWAYS |

### 4) Return: Program existed and return the Alias

Return code:

7E 02 00 01 00 19 41 4C 41 53 00 00 15 10 BD DA C4 BF 31 20 3C 30 30 32 31 3E 00 DD 4F 00 00 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Frame Head 00 01 ¦&0002 (2)¦ Sign ID 00 19 ¦&0004 (2)¦ Data Length 41 4C 41 53 ¦&0006 (4)¦ Command 00 ¦&000A (1)¦ Return Flag (0/0xaa) 00 15 ¦&000B (2)¦ Curr File ID 10 ¦&000D (1)¦ Length (0/16) BD DA C4 BF 31 20 3C 30 30 32 31 3E 00 DD 4F 00 ¦&000E (16)¦ ALIAS 00 ¦&001E (1)¦ Hiding FF FF ¦&001F (2)¦ CRC 7E 00 ¦&0021 (2)¦ Frame Reer | “ALAS” 0 is OK 01 is Hiding File |

### 5) Return: No more program

Return code:

7E 02 00 01 00 08 41 4C 41 53 AA 03 FF 00 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 02 ¦&0022 (2)¦ Frame Head 00 01 ¦&0024 (2)¦ Sign ID 00 08 ¦&0026 (2)¦ Data Length 41 4C 41 53 ¦&0028 (4)¦ Command AA ¦&002C (1)¦ Return Flag 0/aa 03 FF ¦&002D (2)¦ File ID 00 ¦&000D (1)¦ Alias Length FF FF ¦&001E (2)¦ CRC 7E 00 ¦&0020 (2)¦ Frame Reer | “ALAS” 0xaa, no more program <16 bytes |

### 6) Query Current Running Program Alias

Sending:

7E 01 FE FE 00 08 41 4C 41 53 20 00 FF FF FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head,7E01 FE FE ¦&0002 (2)¦ Sign ID 00 08 ¦&0004 (2)¦ Cmd Length 41 4C 41 53 ¦&0006 (4)¦ Command 20 ¦&000A (1)¦ Sub Command: 00 FF FF ¦&000B (3)¦ Reserve,00,FF,FF FF FF ¦&000E (2)¦ CRC 7E 00 ¦&0010 (2)¦ Frame Reer7E,00 | ALAS 0x20 Find Current Running Alias 0x00ffff for old version |

Return code:

## 6.2 Command DELE: Delete files by File ID Range

Sending: Dele all files between File ID 0 to File ID 100 ( 0-100)

7E 01 FE FE 00 0A 44 45 4C 45 00 FF 00 00 00 64 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&00000 (2)¦ Frame Head FE FE ¦&00002 (2)¦ Sign ID 00 0A ¦&00004 (2)¦ Data Length 44 45 4C 45 ¦&00006 (4)¦ Command 00 ¦&0000A (1)¦ Sub Command FF ¦&0000B (1)¦ Ext 00 00 ¦&0000C (2)¦ Start File ID 00 64 ¦&0000E (2)¦ End File ID FF FF ¦&00010 (2)¦ CRC 7E 00 ¦&00012 (2)¦ Frame Reer | “DELE” 0:Erase,1:Erase & Release Flash 0xff for all file extend 00-100 >1023 is all files |

Return code:

| Code: | Description: |
| --- | --- |

## 6.3 Command DELP: Delete all program by File ID range

It’ll delete all programs in range and also delete all files link to program

Sending: Delete all programs between File ID 0 to File ID 100  ( 0-100)

7E 01 FE FE 00 08 44 45 4C 50 00 00 00 07 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head FE FE ¦&0002 (2)¦ Sign ID 00 08 ¦&0004 (2)¦ Data Length 44 45 4C 50 ¦&0006 (4)¦ Command 00 ¦&000A (1)¦ Sub Command 00 ¦&000B (1)¦ Reserve 00 07 ¦&000C (2)¦ OPGMFile ID FF FF ¦&000E (2)¦ CRC 7E 00 ¦&0010 (2)¦ Frame Reer | “DELP” 0:Erase,1:Release Flash Memory 0 00-1023 |

Return code:

| Code: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Frame Head 00 01 ¦&0002 (2)¦ Sign ID 00 08 ¦&0004 (2)¦ Data Length 44 45 4C 50 ¦&0006 (4)¦ Command 00 ¦&000A (1)¦ ReturnFlag 03 ¦&000B (1)¦ Num of deleted 00 1E ¦&000C (2)¦ OPGMFile ID FF FF ¦&000E (2)¦ CRC 7E 00 ¦&0010 (2)¦ Frame Reer | DELP 0x00 is OK, and 0xAA is not such file |

## 6.4 Command DALL: Delete all files

Delete all files between Fild ID 0 to File ID 599, but don’t delete the files 600-1023

DALL Command = DELE (0-599)

Sending:

7E 01 FE FE 00 04 44 41 4C 4C FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&00000 (2)¦ Frame Head FE FE ¦&00002 (2)¦ Sign ID 00 04 ¦&00004 (2)¦ Data Length 44 41 4C 4C ¦&00006 (4)¦ Command FF FF ¦&0000A (2)¦ CRC 7E 00 ¦&0000C (2)¦ Frame Reer | “DALL” |

Return code:

| Code: | Description: |
| --- | --- |

## 6.5 Command GETX: Return a valid file ID

Sometimes, you want to know a valid empty File ID in LED Sign, when you use INST, OGIF command, you can use GETX command

GETX ( Start of range, End of range, Empty number)

Sending:

7E 01 FE FE 00 0A 47 45 54 58 05 00 00 C8 02 58 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head FE FE ¦&0002 (2)¦ Sign ID 00 0A ¦&0004 (2)¦ Data Length 47 45 54 58 ¦&0006 (4)¦ Command 05 ¦&000A (1)¦ File ID Jump Value 00 ¦&000B (1)¦ Reserve 00 C8 ¦&000C (2)¦ Start of Range 02 58 ¦&000E (2)¦ End of Range FF FF ¦&0010 (2)¦ CRC 7E 00 ¦&0012 (2)¦ Frame Reer | “GETX” 0,5,10… File ID 200 File ID 600 |

Return code:

| Code: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Frame Head 00 01 ¦&0002 (2)¦ Sign ID 00 08 ¦&0004 (2)¦ Data Length 47 45 54 58 ¦&0006 (4)¦ Command 00 ¦&000A (1)¦ Return Flag 05 ¦&000B (1)¦ File ID Jump Value 00 D2 ¦&000C (2)¦ Valid File ID FF FF ¦&000E (2)¦ CRC 7E 00 ¦&0010 (2)¦ Frame Reer | ”GETX” 0: OK 7:Not empty file |

## 6.6 Command RSET: Reset the display

Sending:

7E 01 FE FE 00 04 52 53 45 54 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&00000 (2)¦ Frame Head FE FE ¦&00002 (2)¦ Sign ID 00 04 ¦&00004 (2)¦ Data Length 52 53 45 54 ¦&00006 (4)¦ Command FF FF ¦&0000A (2)¦ CRC 7E 00 ¦&0000C (2)¦ Frame Reer | “RSET” |

Return code:

| Code: | Description: |
| --- | --- |

## 6.7 Command FMAT: Initial the display

Sending:

7E 01 FE FE 00 04 46 4D 54 41 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&00000 (2)¦ Frame Head FE FE ¦&00002 (2)¦ Sign ID 00 04 ¦&00004 (2)¦ Data Length 46 4D 54 41 ¦&00006 (4)¦ Command FF FF ¦&0000A (2)¦ CRC 7E 00 ¦&0000C (2)¦ Frame Reer | “FMAT” |

Return code:

| Code: | Description: |
| --- | --- |

## 6.8 Command TIME: Sending Current Time

Sending:

7E 01 FE FE 00 0E 54 49 4D 45 01 08 12 01 01 13 25 21 07 00 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&00000 (2)¦ Frame Head FE FE ¦&00002 (2)¦ Sign ID 00 0E ¦&00004 (2)¦ Data Length 54 49 4D 45 ¦&00006 (4)¦ Command 01 ¦&0000A (1)¦ Sub Command 08 ¦&0000B (1)¦ Time Area 12 01 01 13 25 21 07 ¦&0000C (7)¦ Time 00 ¦&00013 (1)¦ Reserve FF FF ¦&00014 (2)¦ CRC 7E 00 ¦&00016 (2)¦ Frame Reer | “TIME” 01 to set time BCD Code:Y,M,D,M,H,S,Week |

Return code:

| Code: | Description: |
| --- | --- |

## 6.9 Command PWON: Software Power On/Off

### 1) Sending Power Schedule

Sending: PWON on/off and download the Power Schedule

7E 01 FE FE 00 0B 50 57 4F 4E 02 00 00 00 00 00 00 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head,7E01 FE FE ¦&0002 (2)¦ Sign ID 00 0B ¦&0004 (2)¦ Cmd Length 50 57 4F 4E ¦&0006 (4)¦ Command 02 ¦&000A (1)¦ Sub Command 00 00 00 00 00 00 ¦&000B (6)¦ Power Schedule FF FF ¦&0011 (2)¦ CRC 7E 00 ¦&0013 (2)¦ Frame Reer7E,00 | “PWON” 1 Pwr Schedule, 2 PwrOn,3 PwrOff Valid while Sub Command = 1 |

Return code:

7E 02 00 01 00 06 50 57 4F 4E 00 01 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Frame Head 00 01 ¦&0002 (2)¦ Sign ID 00 06 ¦&0004 (2)¦ Data Length 50 57 4F 4E ¦&0006 (4)¦ Command 00 ¦&000A (1)¦ Return Flag 01 ¦&000B (1)¦ Power Status FF FF ¦&000C (2)¦ CRC 7E 00 ¦&000E (2)¦ Frame Reer | Status: 0 Power Off, 1 Power On |

### 2) Get Power Status, and the schedule of Power

Sending:

7E 01 FE FE 00 0B 50 57 4F 4E 00 00 00 00 00 00 00 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head,7E01 FE FE ¦&0002 (2)¦ Sign ID 00 0B ¦&0004 (2)¦ Cmd Length 50 57 4F 4E ¦&0006 (4)¦ Command 00 ¦&000A (1)¦ Sub Command 00 00 00 00 00 00 ¦&000B (6)¦ Power Schedule FF FF ¦&0011 (2)¦ CRC 7E 00 ¦&0013 (2)¦ Frame Reer7E,00 | “PWON” 0 Read Status Reserve |

Return code:

7E 02 00 01 00 0B 50 57 4F 4E 00 01 00 00 00 00 00 00 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 02 ¦&0010 (2)¦ Frame Head,7E02 00 01 ¦&0012 (2)¦ Sign ID 00 0B ¦&0014 (2)¦ Cmd Length 50 57 4F 4E ¦&0016 (4)¦ Command 00 ¦&001A (1)¦ Return Flag 01 ¦&001B (1)¦ Power Status 00 00 00 00 00 00 ¦&001C (6)¦ Power Schedule FF FF ¦&0022 (2)¦ CRC 7E 00 ¦&0024 (2)¦ Frame Reer7E,00 | Status: 0 Power Off, 1 Power On Return Schedule |

The Power Schedule structure is like this:

struct

{

uchar PowerEnable;  //b7-b2(xxxxxx), b1(PowerOnOff), b0(PowerWeekEnable):  1 Enabled

uchar PowerWeek;   //bit7: Use?? bit6-bit0, Monday…Sunday

uchar PowerOn[2];   // (Hour:Minute)

uchar PowerOff[2];   // (Hour:Minute)

}PowerTimer;

## 6.10 Command BRIG: Adjust the brightness (8 scales)

Sending: BRIG

7E 01 FE FE 00 14 42 52 49 47 04 01 00 00 00 00 00 00 00 00 00 00 00 00 00 00 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&00000 (2)¦ Frame Head FE FE ¦&00002 (2)¦ Sign ID 00 14 ¦&00004 (2)¦ Data Length 42 52 49 47 ¦&00006 (4)¦ Command 04 ¦&0000A (1)¦ Sub Command 01 ¦&0000B (1)¦ Bright Running 00 ¦&0000C (1)¦ Reverse 00 ¦&0000D (1)¦ Brightness 00 00 00 00 00 00 00 00 00 00 00 00 ¦&0000E (C)¦ Bright Schedule FF FF ¦&0001A (2)¦ CRC 7E 00 ¦&0001C (2)¦ Frame Reer | “BRIG” 0 Manual / 1 Auto / 2 Schedule |

Return code:

7E 02 00 01 00 06 42 52 49 47 00 00 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 02 ¦&00000 (2)¦ Frame Head 00 01 ¦&00002 (2)¦ Sign ID 00 06 ¦&00004 (2)¦ Data Length 42 52 49 47 ¦&00006 (4)¦ Command 00 ¦&0000A (1)¦ Return Flag 00 ¦&0000B (1)¦ Reserve FF FF ¦&0000C (2)¦ CRC 7E 00 ¦&0000E (2)¦ Frame Reer |  |

## 6.11 Command QVER: Query the version of firmware

## 6.12 Command GIMG: Return Current Display Image from sign

GIMG Command: Get the Current Display Image of Sign

7E 01 FE FE 00 0A 47 49 4D 47 A0 70 00 00 00 12 FF FF 7E 00

| Code | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head FE FE ¦&0002 (2)¦ Sign ID 00 0A ¦&0004 (2)¦ Data Length 47 49 4D 47 ¦&0006 (4)¦ Command A0 ¦&000A (1)¦ Reserve 70 ¦&000B (1)¦ Color Page 00 00 ¦&000C (2)¦ Start Col 00 12 ¦&000E (2)¦ Return Width Num. FF FF ¦&0010 (2)¦ CRC 7E 00 ¦&0012 (2)¦ Frame Reer | “GIMG” R:0x40, R/G: 0x60,R/G/B:0x70 |

Return code:

7E 02 00 01 00 E6 47 49 4D 47 00 00 01 70 00 00 00 12 00 D8 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 01 01 01 01 02 02 02 02 04 04 04 04 08 08 08 08 10 10 10 10 20 20 20 20 40 40 40 40 80 80 80 80 01 01 01 01 02 02 02 02 04 04 04 04 08 08 08 08 10 10 10 10 20 20 20 20 40 40 40 40 80 80 80 80 01 01 01 01 02 02 02 02 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 FF FF 7E 00

| Code | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Frame Head 00 01 ¦&0002 (2)¦ Sign ID 00 E6 ¦&0004 (2)¦ Data Length 47 49 4D 47 ¦&0006 (4)¦ Command 00 ¦&000A (1)¦ Return Flag 00 ¦&000B (1)¦ Reserve 00 01 ¦&000C (1)¦ Height Word 70 ¦&000D (1)¦ Color Page 00 00 ¦&000E (2)¦ Start Col 00 12 ¦&0010 (2)¦ Return Width Num. 00 D8 ¦&0012 (2)¦ Data Length 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 ¦&0014 (72)¦ Data R 01 01 01 01 02 02 02 02 04 04 04 04 08 08 08 08 10 10 10 10 20 20 20 20 40 40 40 40 80 80 80 80 01 01 01 01 02 02 02 02 04 04 04 04 08 08 08 08 10 10 10 10 20 20 20 20 40 40 40 40 80 80 80 80 01 01 01 01 02 02 02 02 ¦&005C (72)¦ Data G 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 ¦&00A4 (72)¦ DataB FF FF ¦&00EC (2)¦ CRC 7E 00 ¦&00EE (2)¦ Frame Reer | GIMG 00: OK 1 Word = 4 Bytes =32 Pixels R:0x40, R/G: 0x60,R/G/B:0x70 = Width*Bytes*Pages First Page Data Second Page Data Third Page Data |

First Time,  Get data from col 0 to col 47,

7E 01 FE FE 00 0A 47 49 4D 47 A0 40 00 00 00 30 FF FF 7E 00

Second Time, Get data from col 48 to 95…

7E 01 FE FE 00 0A 47 49 4D 47 A0 40 00 30 00 30 FF FF 7E 00

## 6.13 Command STID: Setup ID & Other functions

### 0) 0x00 Query Sign ID and Sign MAC Address

| Code | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head FE FE ¦&0002 (2)¦ Sign ID 00 0A ¦&0004 (2)¦ Data Length 53 54 49 44 ¦&001B (4)¦ cmd STID 00 ¦&000A (1)¦ CMD 0/1/10/11/21-24 00 00 00 00 ¦&000B (4)¦ MAC 00 00 ¦&000F (2)¦ ID FF FF ¦&0011 (2)¦ CRC 7E 00 ¦&0013 (2)¦ 帧尾 | 查询ID与MAC地址 |

### 1) 0x01 Setup ID according to MAC address

| Code | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head FE FE ¦&0002 (2)¦ Sign ID 00 0A ¦&0004 (2)¦ Data Length 53 54 49 44 ¦&001B (4)¦ cmd STID 10 ¦&001F (1)¦ CMD 1/10/11/21-24 00 00 00 00 ¦&0020 (4)¦ MAC 00 00 ¦&0024 (2)¦ ID FF FF ¦&0026 (2)¦ CRC 7E 00 ¦&0028 (2)¦ Frame Tail | 写ID号 只有MAC地址相符才可以写ID 要写的ID号 |

### 2) 0x11 Enter Sign ID Manual Setup Function

7E 01 FE FE 00 0B 53 54 49 44 11 00 00 00 00 00 00 FF FF 7E 00

| Code | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head FE FE ¦&0002 (2)¦ Sign ID 00 0B ¦&0004 (2)¦ Data Length 53 54 49 44 ¦&001B (4)¦ cmd STID 11 ¦&0034 (1)¦ CMD 0/1/10/11/21-24 00 00 00 00 ¦&0035 (4)¦ MAC 00 00 ¦&0039 (2)¦ ID FF FF ¦&003B (2)¦ CRC 7E 00 ¦&0028 (2)¦ Frame Tail |  |

### 3) 0x21 Enter Timer Function

7E 01 FE FE 00 05 53 54 49 44 21 FF FF 7E 00

| Code | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head FE FE ¦&0002 (2)¦ Sign ID 00 05 ¦&0004 (2)¦ Data Length 53 54 49 44 ¦&001B (4)¦ cmd STID 21 ¦&0034 (1)¦ CMD 0/1/10/11/21-24 FF FF ¦&003B (2)¦ CRC 7E 00 ¦&0028 (2)¦ Frame Tail |  |

### 4) 0x22 Timer START

7E 01 FE FE 00 05 53 54 49 44 22 FF FF 7E 00

| Code | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head FE FE ¦&0002 (2)¦ Sign ID 00 05 ¦&0004 (2)¦ Data Length 53 54 49 44 ¦&001B (4)¦ cmd STID 22 ¦&0034 (1)¦ CMD 0/1/10/11/21-24 FF FF ¦&003B (2)¦ CRC 7E 00 ¦&0028 (2)¦ Frame Tail |  |

### 5) 0x23 Timer PAUSE

7E 01 FE FE 00 05 53 54 49 44 23 FF FF 7E 00

| Code | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head FE FE ¦&0002 (2)¦ Sign ID 00 05 ¦&0004 (2)¦ Data Length 53 54 49 44 ¦&001B (4)¦ cmd STID 23 ¦&0034 (1)¦ CMD 0/1/10/11/21-24 FF FF ¦&003B (2)¦ CRC 7E 00 ¦&0028 (2)¦ Frame Tail |  |

### 6) 0x24 Timer STOP/CLEAR

7E 01 FE FE 00 05 53 54 49 44 24 FF FF 7E 00

| Code | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head FE FE ¦&0002 (2)¦ Sign ID 00 05 ¦&0004 (2)¦ Data Length 53 54 49 44 ¦&001B (4)¦ cmd STID 24 ¦&0034 (1)¦ CMD 0/1/10/11/21-24 FF FF ¦&003B (2)¦ CRC 7E 00 ¦&0028 (2)¦ Frame Tail |  |

### 7) 0x10 Exit Function of Setup ID or Timer

7E 01 FE FE 00 0B 53 54 49 44 10 00 00 00 00 00 00 FF FF 7E 00

| Code | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head FE FE ¦&0002 (2)¦ Sign ID 00 0B ¦&0004 (2)¦ Data Length 53 54 49 44 ¦&001B (4)¦ cmd STID 10 ¦&0034 (1)¦ CMD 0/1/10/11/21-24 00 00 00 00 ¦&0035 (4)¦ MAC 00 00 ¦&0039 (2)¦ ID FF FF ¦&003B (2)¦ CRC 7E 00 ¦&0028 (2)¦ Frame Tail |  |

## 6.14 Command BEEP: Beep or Turn On/Off beep

7E 01 FE FE 00 06 42 45 45 50 00 64 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head FE FE ¦&0002 (2)¦ Sign ID 00 0A ¦&0004 (2)¦ Data Length 42 45 45 50 ¦&0006 (4)¦ Command BEEP 00 ¦&000A (1)¦ Beep Mode 64 ¦&000B (1)¦ Time: ms FF FF ¦&000C (2)¦ CRC 7E 00 ¦&000E (2)¦ 帧尾 | 0: Normal ms / 1:Turn on / 2: Turn off / 3:Return Beep |

Return: 7E 02 00 01 00 06 42 45 45 50 00 00 FF FF 7E 00

| Code: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Frame Head FE FE ¦&0002 (2)¦ Sign ID 00 0A ¦&0004 (2)¦ Data Length 42 45 45 50 ¦&0006 (4)¦ Command BEEP 00 ¦&000A (1)¦ ReturnFlag 00 ¦&000B (1)¦ Beep Status FF FF ¦&000C (2)¦ CRC 7E 00 ¦&000E (2)¦ Rear | 00,Normal 00: Normal 01:Turn on |

# 7. Feature and Escape String

## 7.1 How to display existed image in text string

In a text string, the code start of 0x80, and the second byte is in the range of 0x10-0x1f are index code of font matrix library.

First Code Byte,  0x80 – 0xff

Seconds Code Byte,  0x10 – 0x1f

e.g.

Insert First Font Matrix, 0x80, 0x10

Insert Second Font Matrix,0x80, 0x11

…

Insert 16th Font Matrix, 0x80,0x1f

Insert 17th Font Matrix, 0x81,0x10

Insert 18th Font Matrix, 0x81,0x11

…

## 7.2 Normal Escape String

| Escape String | Function | Others |
| --- | --- | --- |
| [FL000] | Font Matrix Library ID | [FL000] [FL0C8] |
| [CH8] | Height Pixels of Lines | [CH8] [CH12] [CH16] |
| [CR] | Return |  |
| [CXC] | Center [CXC], Left[CXL], Right[CXR] | Align |
| [CV] | FLASH Memory Variable |  |
| [Cv] | RAM Memory Variable |  |
| [DY] | Year |  |
| [DL] | Month |  |
| [DD] | Date |  |
| [DH] | Hour |  |
| [DM] | Minute |  |
| [DS] | Second |  |
| [DW] | Week |  |
| [DN] | Chinese Lunar |  |

## 7.3 Color Code and Color Escape String

| Escape String | Color Code BYTE |  | Font Color Name: English |
| --- | --- | --- | --- |
| [FC00] [FC01] [FC02] [FC03] [FC07] [FC08] [FC09] [FC0D] | 0x00 0x01 0x02 0x03 0x04 0x05 0x06 0x07 |  | 0=DEFAULT 1=RED 2=GREEN 3=YELLOW 4=BLUE 5=PINK 6=CYAN 7=WHITE |

* Notice: [FC00] – [FC0F] is max 16 colors, but Color Code is max 8 colors

## 7.4 Action Code

| Escape String | Action Code BYTE | Action Name: English |  |
| --- | --- | --- | --- |
| None | 0x00 0x01 0x02 0x03 0x04 0x05 0x06 0x07 0x08 0x09 0x0a 0x0b 0x0c 0x0d 0x0e 0x0f 0x10 | AUTO HOLD ROTATE ROTATE RIGHT SHUTTER SNOW SPARKLE TRIANGLE WIDE ROLL LEFT ROLL RIGHT ROLL UP ROLL DOWN WIPE LEFT WIPE RIGHT WIPE DOWN WIPE UP |  |

## 7.5 Asian/English Font ID Escape String

* Means it maybe not exist

| Escape String | Font Code BYTE[2] | Font Name | Charset |
| --- | --- | --- | --- |
| [FT100] [FT101] [FT102] [FT108] [FT110] [FT111] [FT112] [FT118] [FT120] [FT121] | 0x0100 0x0101 0x0102 0x0108 0x0110 0x0111 0x0112 0x0118 0x0120 0x0121 | 7x5 Sans Serif 7x6 Sans Serif 7x7 Sans Serif 12x8 EFont 16x8 Normal 16x8 Thin 16x8 Thick 16x8 Digit * 14x12 PMincho * 16x8 Mincho | Western |
| [FT200] [FT210] [FT300] [FT310] | 0x0200 0x0210 0x0300 0x0310 | SongTi 12 SongTi 16 * Tranditional 12 * Tranditional 16 | Aisian |
| [FT500] | 0x0500 | Korean Code Page |  |

## 7.6 Stay Time Code

| Escape String | Code (BYTE) | Stay Seconds | Description |
| --- | --- | --- | --- |
| None | 0x00 0x01 0x02 0x03 0x04 0x05 0x06 0x07 0x08 0x09 0x0a 0x0b 0x0c 0x0d 0x0e | 0.5s(Default) 0s 0.02s 0.04s 0.10s 0.25s 0.50s 0.75s 1s 2s 3s 5s 7.5s 10s 15s |  |

End of file.