C32S01 LED Moving Sign Controller

Communication Protocol

V1.23

1. Overview of Event-Driven	2

1.1 How to use Event-Driven	2

1.2 Display Memory System	2

1.3 Event-Driven Command types	3

1.4 Devemopment Tools	3

2. The Protocol of Event-Driven	4

2.1 CMD=1 Enter Event-Driven Status and Initial Memory	4

2.2 CMD=0 Exit the Event-Driven Status	5

2.3 CMD=2 Clear Screen of Special Area	6

2.4 CMD=3  Display from Memory B	7

2.4 CMD=4 Roll Memory B to Display Memory	9

2.5 CMD=5 Display Memory Flash or Back Color Flash	10

2.6 CMD=6 Print Text or Image in Areas or Windows	11

2.6.1 Print Text to Display Area	11

2.6.2 Print Text to Windows	12

2.6.3 Same Return Value	13

2.7 CMD=7 Start to Run All Windows from Back Windows	14

2.8 CMD=8 Clear all data in Back Windows	15

2.9 CMD=9 LINK to an Existed Program	16

2.10 Will be come: Print Image	17

3. Feature and Escape String	17

3.1 How to display existed image in text string	17

3.2 Normal Escape String	18

3.3 Color Code and Color Escape String	18

3.4 Action Code	19

3.5 Asian/English Font ID Escape String	19

3.6 Stay Time Code	20

# 1. Overview of Event-Driven

## 1.1 How to use Event-Driven

Event-driven state, is also a part of the secondary development protocol, which is a real-time response state of the LED display. Therefore, when using the event-driven protocol, you can also use the traditional secondary development protocol. LED display event-driven, that is, the LED display as a display panel, when and where display message is according to the instructions of the control machine,  LED display will remain in the current display state waiting for command if the control machine does not send command.

In the traditional protocol, the variable sending protocol VRAM / VROM, the program OTXT protocol, the OGIF animation protocol, the on-screen content GIMG protocol, etc., can be used in event-driven, combined with the event protocol of this article, can form a very flexible Application combination.

Secondary Development Protocol

Event-Driven Protocol

## 1.2 Display Memory System

Before the control machine controls the operation of the LED display, make sure that the LED display is in the event-driven state. The LED display has the normal display state and the event-driven state, which can be switched by command. The nomal display will be show when power on.

In LED display memory, there are two display areas, one is the current display area, called "Display Area", the other is the buffer of the display area, in fact, do not show up, called "Background Display Area" or “Display Buffer“.

There’re 7 windows can be display in LED display, and they also have 2 memorys, one is current display area, the other is data buffer memory.

.

## 1.3 Event-Driven Command types

Actual control, the instructions will be divided into the following classes:

Class A: Screen control: such as clear screen, foreground movement, press the action disappeared, flashing and so on.

Class B: Print,  print content in the foreground / background location, such as text, pictures.

Class C: Display, the display (Display), the background display area of the content, in accordance with the specified action, moved to the foreground to display.

Class D: Link, if the display is active, such as string movement, animation, multi-page display etc., must work as program mode

Class E: auxiliary instructions such as power down, save prospects, and restore foreground instructions

## 1.4 Devemopment Tools

We provide a sofrware, for customers to make simulation of secondary development.

# 2. The Protocol of Event-Driven

## 2.1 CMD=1 Enter Event-Driven Status and Initial Memory

Command Function:

1) Normal program state, this instruction causes the display to enter the online event control mode and clear the foreground, background, and 7 Window's contents

2) Event state, the implementation of this directive, the equivalent of the initial clear operation. Clear the foreground, background, and 7 Window's content

Note: The program ID must be set to 0x400 or above to enter the event online state, below 0x400 for the program protocol.

Send:  7E 01 FE FE 00 08 45 56 45 4E 01 02 04 00 FF FF 7E 00

| Command Bytes: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Head of Frame FE FE ¦&0002 (2)¦ Screen ID 00 08 ¦&0004 (2)¦ Data Length 45 56 45 4E ¦&0006 (4)¦ Cmd EVEN 01 ¦&000A (1)¦ Sub Cmd ID 02 ¦&000B (1)¦ Length 04 00 ¦&000C (2)¦ @Program ID FF FF ¦&000E (2)¦ CRC 7E 00 ¦&0010 (2)¦ EOF | FEFE IS SINGLE SIGN MODE 1:Enter Event-Driven status Pgm ID>0x400 Enter Event-Driven |

Return String: 7E 02 00 01 00 07 45 56 45 4E 00 01 55 FF FF 7E 00

| Command Bytes: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Head of Frame 00 01 ¦&0002 (2)¦ Screen ID 00 07 ¦&0004 (2)¦ Data Length 45 56 45 4E ¦&0006 (4)¦ Cmd EVEN 00 ¦&000A (1)¦ Return Flag 01 ¦&000B (1)¦ Sub Cmd ID 55 ¦&000C (1)¦ Event Status 55 FF FF ¦&000D (2)¦ CRC 7E 00 ¦&000F (2)¦ EOF | Return Sign ID 00 right,0xaa error 0x55 is in event status |

## 2.2 CMD=0 Exit the Event-Driven Status

Command Function:

This Command will cause the display to exit the event status and return to the program or on demand

Send:  7E 01 FE FE 00 06 45 56 45 4E 00 00 FF FF 7E 00

| Command: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Head of Frame FE FE ¦&0002 (2)¦ Screen ID 00 06 ¦&0004 (2)¦ Data Length 45 56 45 4E ¦&0006 (4)¦ Cmd EVEN 00 ¦&000A (1)¦ Sub Cmd ID 00 ¦&000B (1)¦ Length FF FF ¦&000C (2)¦ CRC 7E 00 ¦&000E (2)¦ EOF | FEFE IS SINGLE SIGN MODE 00 |

Return String: 7E 02 00 01 00 07 45 56 45 4E 00 00 00 FF FF 7E 00

| Command Bytes: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Head of Frame 00 01 ¦&0002 (2)¦ Screen ID 00 07 ¦&0004 (2)¦ Data Length 45 56 45 4E ¦&0006 (4)¦ Cmd EVEN 00 ¦&000A (1)¦ Return Flag 00 ¦&000B (1)¦ Sub Cmd ID 00 ¦&000C (1)¦ Event Status 55 FF FF ¦&000D (2)¦ CRC 7E 00 ¦&000F (2)¦ EOF | Return Sign ID 00 right,0xaa error 0x55 is in event status |

## 2.3 CMD=2 Clear Screen of Special Area

Command Function:

Clear the contents of the desktop after the front desk, note that this command can’t be used to clear the contents of the background window. Note that when the width is 0, it means to the right of the screen, when the height is 0, that up to the bottom of the screen. Note that the width value is the short type, that is, the low byte is preceding, the high byte is in the post

Send:  7E 01 FE FE 00 0D 45 56 45 4E 02 07 00 00 40 00 00 10 00 FF FF 7E 00

| Command Bytes: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Head of Frame FE FE ¦&0002 (2)¦ Screen ID 00 0D ¦&0004 (2)¦ Data Length 45 56 45 4E ¦&0006 (4)¦ Cmd EVEN 02 ¦&000A (1)¦ Sub Cmd ID 07 ¦&000B (1)¦ Length 00 00 ¦&000C (2)¦ X POSITION 40 00 ¦&000E (2)¦ W WIDTH PIXELS 00 ¦&0010 (1)¦ Y POSITION 10 ¦&0011 (1)¦ H HEIGHT PIXELS 00 ¦&0012 (1)¦ Memory Position FF FF ¦&0013 (2)¦ CRC 7E 00 ¦&0015 (2)¦ EOF | FEFE IS SINGLE SIGN MODE Clear Screen Area Area 40 00 is 64 Pixels Area Area A:00,B:01,A+B:02 |

Return String: 7E 02 00 01 00 07 45 56 45 4E 00 02 55 FF FF 7E 00

| Command Bytes: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Head of Frame 00 01 ¦&0002 (2)¦ Screen ID 00 07 ¦&0004 (2)¦ Data Length 45 56 45 4E ¦&0006 (4)¦ Cmd EVEN 00 ¦&000A (1)¦ Return Flag 02 ¦&000B (1)¦ Sub Cmd ID 55 ¦&000C (1)¦ Event Status 55 FF FF ¦&000D (2)¦ CRC 7E 00 ¦&000F (2)¦ EOF | Return Sign ID 00 right,0xaa error 0x55 is in event status |

## 2.4 CMD=3  Display from Memory B

Command Function:

According to the action requirements, move the contents of B to memory A and display. Note that this function applies to the desktop after the desk handling, not suitable for the output of the window content.

The reason for this function is because if you print directly in the memory A, because of the time difference, and if you use the direct print memory A function, you can’t select the action, can’t be centered. This function can make printed memory B, Immediately moved to A show out, or with a certain action displayed.

Send:  7E 01 FE FE 00 0E 45 56 45 4E 03 08 00 00 40 00 00 10 00 02 FF FF 7E 00

| Command Bytes: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Head of Frame FE FE ¦&0002 (2)¦ Screen ID 00 0E ¦&0004 (2)¦ Data Length 45 56 45 4E ¦&0006 (4)¦ Cmd EVEN 03 ¦&000A (1)¦ Sub Cmd ID 08 ¦&000B (1)¦ Length 00 00 ¦&000C (2)¦ X POSITION 40 00 ¦&000E (2)¦ W WIDTH PIXELS 00 ¦&0010 (1)¦ Y POSITION 10 ¦&0011 (1)¦ H HEIGHT PIXELS 00 ¦&0012 (1)¦ Reserved 02 ¦&0013 (1)¦ Display Action FF FF ¦&0014 (2)¦ CRC 7E 00 ¦&0016 (2)¦ EOF | FEFE IS SINGLE SIGN MODE Display Memory Area Area 40 00 is 64 Pixels Area Area 02 is rotate |

Return String: 7E 02 00 01 00 07 45 56 45 4E 00 03 55 FF FF 7E 00

| Command Bytes: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Head of Frame 00 01 ¦&0002 (2)¦ Screen ID 00 07 ¦&0004 (2)¦ Data Length 45 56 45 4E ¦&0006 (4)¦ Cmd EVEN 00 ¦&000A (1)¦ Return Flag 03 ¦&000B (1)¦ Sub Cmd ID 55 ¦&000C (1)¦ Event Status 55 FF FF ¦&000D (2)¦ CRC 7E 00 ¦&000F (2)¦ EOF | Return Sign ID 00 right,0xaa error 0x55 is in event status |

## 2.4 CMD=4 Roll Memory B to Display Memory

Command Function:

This command combine the frontdesk and background as a unit, move foreground, and fill the foreground space with the background data, the formation of background data to the foreground effect.

Send:  7E 01 FE FE 00 09 45 56 45 4E 04 03 10 01 00 FF FF 7E 00

| Command Bytes: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Head of Frame FE FE ¦&0002 (2)¦ Screen ID 00 09 ¦&0004 (2)¦ Data Length 45 56 45 4E ¦&0006 (4)¦ Cmd EVEN 04 ¦&000A (1)¦ Sub Cmd ID 03 ¦&000B (1)¦ Length 10 ¦&000C (1)¦ Move Pixels 01 ¦&000D (1)¦ Use Buffer Data? 01 ¦&000E (1)¦ Roll Direction FF FF ¦&000F (2)¦ CRC 7E 00 ¦&0011 (2)¦ EOF | FEFE IS SINGLE SIGN MODE Roll Screen 0x10 is 16 Pixel-Lines 0: Yes, 1:No use buffer data 0:Roll Up, 1:Roll Down |

Return String:  7E 02 00 01 00 07 45 56 45 4E 00 04 55 FF FF 7E 00

| Command Bytes: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Head of Frame 00 01 ¦&0002 (2)¦ Screen ID 00 07 ¦&0004 (2)¦ Data Length 45 56 45 4E ¦&0006 (4)¦ Cmd EVEN 00 ¦&000A (1)¦ Return Flag 04 ¦&000B (1)¦ Sub Cmd ID 55 ¦&000C (1)¦ Event Status 55 FF FF ¦&000D (2)¦ CRC 7E 00 ¦&000F (2)¦ EOF | Return Sign ID 00 right,0xaa error 0x55 is in event status |

## 2.5 CMD=5 Display Memory Flash or Back Color Flash

Command Function:

Make a zone of front desk into flashing, or cancel the flashing

Send:  7E 01 FE FE 00 0E 45 56 45 4E 05 08 00 00 40 00 00 10 00 01 FF FF 7E 00

| Command Bytes: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Head of Frame FE FE ¦&0002 (2)¦ Screen ID 00 0E ¦&0004 (2)¦ Data Length 45 56 45 4E ¦&0006 (4)¦ Cmd EVEN 05 ¦&000A (1)¦ Sub Cmd ID 08 ¦&000B (1)¦ Length 00 00 ¦&000C (2)¦ X POSITION 40 00 ¦&000E (2)¦ W WIDTH PIXELS 00 ¦&0010 (1)¦ Y POSITION 10 ¦&0011 (1)¦ H HEIGHT PIXELS 00 ¦&0012 (1)¦ Reserved 01 ¦&0013 (1)¦ 01 Flash/00 Disable FF FF ¦&0014 (2)¦ CRC 7E 00 ¦&0016 (2)¦ EOF | FEFE IS SINGLE SIGN MODE Flash Area Area Area Area Flash Enable or Disable |

Return String:  7E 02 00 01 00 07 45 56 45 4E 00 05 55 FF FF 7E 00

| Command Bytes: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Head of Frame 00 01 ¦&0002 (2)¦ Screen ID 00 07 ¦&0004 (2)¦ Data Length 45 56 45 4E ¦&0006 (4)¦ Cmd EVEN 00 ¦&000A (1)¦ Return Flag 05 ¦&000B (1)¦ Sub Cmd ID 55 ¦&000C (1)¦ Event Status 55 FF FF ¦&000D (2)¦ CRC 7E 00 ¦&000F (2)¦ EOF | Return Sign ID 00 right,0xaa error 0x55 is in event status |

## 2.6 CMD=6 Print Text or Image in Areas or Windows

### 2.6.1 Print Text to Display Area

Command Function:

Unlike the window printing, Area printing is printed directly on the desktop of front or background. Note that the text level center function, can only be printed in the background to achieve the front desk can not handle the level of center function.

Send:  7E 01 FE FE 00 35 45 56 45 4E 06 2F 00 00 00 40 00 00 10 00 00 00 00 01 00 00 00 00 01 00 02 00 5B 43 48 38 5D 41 42 5B 43 52 5D 43 44 5B 46 43 30 31 5D 5B 46 54 31 30 31 5D 62 FF FF 7E 00

| Command Bytes: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Head of Frame FE FE ¦&0002 (2)¦ Screen ID 00 35 ¦&0004 (2)¦ Data Length 45 56 45 4E ¦&0006 (4)¦ Cmd EVEN 06 ¦&000A (1)¦ Sub Cmd ID 2F ¦&000B (1)¦ Length 00 ¦&000C (1)¦ Reserved 00 00 ¦&000D (2)¦ X POSITION 40 00 ¦&000F (2)¦ W WIDTH PIXELS 00 ¦&0011 (1)¦ Y POSITION 10 ¦&0012 (1)¦ H HEIGHT PIXELS 00 ¦&0013 (1)¦ What Memory area 00 ¦&0014 (1)¦ Rotate Speed 00 ¦&0015 (1)¦ Stay Time 00 ¦&0016 (1)¦ Loop Times 01 ¦&0017 (1)¦ Reserved 00 ¦&0018 (1)¦ Multi-Line Display 00 ¦&0019 (1)¦ Align 00 ¦&001A (1)¦ Default Color 00 ¦&001B (1)¦ Reserved(FontMode) 01 00 ¦&001C (2)¦ Western Font ID 02 00 ¦&001E (2)¦ Asian Font ID 5B 43 48 38 5D 41 42 5B 43 52 5D 43 44 5B 46 43 30 31 5D 5B 46 54 31 30 31 5D 62 ¦&0020 (27)¦ Display Message FF FF ¦&003B (2)¦ CRC 7E 00 ¦&003D (2)¦ EOF | FEFE IS SINGLE SIGN MODE TEXT指令 Area Area Area Area 0:Display area, 1:Back Memory 0: Fast,1.2.3…Slow * Refer the attachment 0: Allways,>0 Times 0:Single Line,1:Multi-Lines 0:Center,1:Left,2:Right * Refer the attachment 0x100 * Refer the attachment 0x200 * Refer the attachment [CH8]AB[CR]CD[FC01][FT101]b |

### 2.6.2 Print Text to Windows

Command Function:

This command will print text to Display Window (Display Immediately) or Back Window(not display)

Notice:

1) If you print to the Display window,it will only interrupt the current window, other Window's process without any impact, there will be no flash.

2) If you use the print to the background window, use the window display command to display at the same time, all the windows start at the same time.

Send:  7E 01 FE FE 00 35 45 56 45 4E 06 2F 02 40 00 40 00 00 10 01 00 00 00 01 00 00 00 00 01 00 02 00 5B 43 48 38 5D 41 42 5B 43 52 5D 43 44 5B 46 43 30 31 5D 5B 46 54 31 30 31 5D 62 FF FF 7E 00

| Command Bytes: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Head of Frame FE FE ¦&0002 (2)¦ Screen ID 00 35 ¦&0004 (2)¦ Data Length 45 56 45 4E ¦&0006 (4)¦ Cmd EVEN 06 ¦&000A (1)¦ Sub Cmd ID 2F ¦&000B (1)¦ Length 02 ¦&000C (1)¦ Windows Number 40 00 ¦&000D (2)¦ X POSITION 40 00 ¦&000F (2)¦ W WIDTH PIXELS 00 ¦&0011 (1)¦ Y POSITION 10 ¦&0012 (1)¦ H HEIGHT PIXELS 01 ¦&0013 (1)¦ Action 00 ¦&0014 (1)¦ Speed of Rotate* 00 ¦&0015 (1)¦ Stay Seconds 00 ¦&0016 (1)¦ Loop Times 01 ¦&0017 (1)¦ Memory Position 00 ¦&0018 (1)¦ Multi Lines Disp 00 ¦&0019 (1)¦ Align 00 ¦&001A (1)¦ Default Color 00 ¦&001B (1)¦ Reserved(Font Mode) 01 00 ¦&001C (2)¦ English Font ID 02 00 ¦&001E (2)¦ Asian Font ID 5B 43 48 38 5D 41 42 5B 43 52 5D 43 44 5B 46 43 30 31 5D 5B 46 54 31 30 31 5D 62 ¦&0020 (27)¦ Display Text FF FF ¦&003B (2)¦ CRC 7E 00 ¦&003D (2)¦ EOF | FEFE IS SINGLE SIGN MODE Print Text Command 1-7,Max is No.7 Windows Area Windows Area Windows Area Windows Area * Refer the attachment 0 Fast,1.2.3…Slow /Action Hold * Refer the attachment 0: Allways, >0 is Times 1:Back Memory,0:Display Memory 0:Single line,1:Multi-Lines 0:Center,1:Left,2:Right * Refer the attachment 0x01 is western 0x100 * Refer the attachment 0x200 * Refer the attachment [CH8]AB[CR]CD[FC01][FT101]b |

### 2.6.3 Same Return Value

Return String:  7E 02 00 01 00 07 45 56 45 4E 00 06 55 FF FF 7E 00

| Command Bytes: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Head of Frame 00 01 ¦&0002 (2)¦ Screen ID 00 07 ¦&0004 (2)¦ Data Length 45 56 45 4E ¦&0006 (4)¦ Cmd EVEN 00 ¦&000A (1)¦ Return Flag 06 ¦&000B (1)¦ Sub Cmd ID 55 ¦&000C (1)¦ Event Status 55 FF FF ¦&000D (2)¦ CRC 7E 00 ¦&000F (2)¦ EOF | Return Sign ID 00 right,0xaa error 0x55 is in event status |

## 2.7 CMD=7 Start to Run All Windows from Back Windows

Command Function:

This command will interrupt current running progress, and mov all data from Back Memory to Display Memory, then running all the program of window

Send:  7E 01 FE FE 00 08 45 56 45 4E 07 02 FF 00 FF FF 7E 00

| Command Bytes: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Head of Frame FE FE ¦&0002 (2)¦ Screen ID 00 08 ¦&0004 (2)¦ Data Length 45 56 45 4E ¦&0006 (4)¦ Cmd EVEN 07 ¦&000A (1)¦ Sub Cmd ID 02 ¦&000B (1)¦ Length FF ¦&000C (1)¦ Reserved(Wnd#) 00 ¦&000D (1)¦ Reserved FF FF ¦&000E (2)¦ CRC 7E 00 ¦&0010 (2)¦ EOF | FEFE IS SINGLE SIGN MODE Display in Back Memory 0xff means all windows |

Return String:  7E 02 00 01 00 07 45 56 45 4E 00 07 55 FF FF 7E 00

| Command Bytes: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Head of Frame 00 01 ¦&0002 (2)¦ Screen ID 00 07 ¦&0004 (2)¦ Data Length 45 56 45 4E ¦&0006 (4)¦ Cmd EVEN 00 ¦&000A (1)¦ Return Flag 07 ¦&000B (1)¦ Sub Cmd ID 55 ¦&000C (1)¦ Event Status 55 FF FF ¦&000D (2)¦ CRC 7E 00 ¦&000F (2)¦ EOF | Return Sign ID 00 right,0xaa error 0x55 is in event status |

## 2.8 CMD=8 Clear all data in Back Windows

Command Function:

This command only clear the data in Back Windows, it’s different from the Initial Command, which will clear all data in Back Windows and Display Windows.  Total 7 windows can be used in this system.

Notice: This command can not interrupt the current running progress, and the Initial Command will interrupt the running program immediately. They are different.

Send:  7E 01 FE FE 00 07 45 56 45 4E 08 01 FF FF FF 7E 00

| Command Bytes: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Head of Frame FE FE ¦&0002 (2)¦ Screen ID 00 07 ¦&0004 (2)¦ Data Length 45 56 45 4E ¦&0006 (4)¦ Cmd EVEN 08 ¦&000A (1)¦ Sub Cmd ID 01 ¦&000B (1)¦ Length FF ¦&000C (1)¦ Reserved(Wnd#) FF FF ¦&000D (2)¦ CRC 7E 00 ¦&000F (2)¦ EOF | FEFE IS SINGLE SIGN MODE Clear all back windows 0xff means all windows |

Return String:  7E 02 00 01 00 07 45 56 45 4E 00 08 55 FF FF 7E 00

| Command Bytes: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Head of Frame 00 01 ¦&0002 (2)¦ Screen ID 00 07 ¦&0004 (2)¦ Data Length 45 56 45 4E ¦&0006 (4)¦ Cmd EVEN 00 ¦&000A (1)¦ Return Flag 08 ¦&000B (1)¦ Sub Cmd ID 55 ¦&000C (1)¦ Event Status 55 FF FF ¦&000D (2)¦ CRC 7E 00 ¦&000F (2)¦ EOF | Return Sign ID 00 right,0xaa error 0x55 is in event status |

## 2.9 CMD=9 LINK to an Existed Program

Command Function:

It’s like the function of OLE, You can link a window to an existed program, it’ll display all messages in first window of program.  It can be text, image, animation…

Send: 7E 01 FE FE 00 35 45 56 45 4E 06 2F 02 40 00 40 00 00 10 01 00 00 00 01 00 00 00 00 01 00 02 00 5B 43 48 38 5D 41 42 5B 43 52 5D 43 44 5B 46 43 30 31 5D 5B 46 54 31 30 31 5D 62 FF FF 7E 00

| Command Bytes: | Description: |
| --- | --- |
| 7E 01 ¦&0000 (2)¦ Head of Frame FE FE ¦&0002 (2)¦ Screen ID 00 1A ¦&0004 (2)¦ Data Length 45 56 45 4E ¦&0006 (4)¦ Cmd EVEN 09 ¦&000A (1)¦ Sub Cmd ID 14 ¦&000B (1)¦ Length 01 ¦&000C (1)¦ Windows Number 00 00 ¦&000D (2)¦ X POSITION 40 00 ¦&000F (2)¦ W WIDTH PIXELS 00 ¦&0011 (1)¦ Y POSITION 10 ¦&0012 (1)¦ H HEIGHT PIXELS 01 ¦&0013 (1)¦ x 00 ¦&0014 (1)¦ x 00 ¦&0015 (1)¦ x 00 ¦&0016 (1)¦ x 01 ¦&0017 (1)¦ x 00 ¦&0018 (1)¦ x 00 ¦&0019 (1)¦ x 00 ¦&001A (1)¦ x 00 ¦&001B (1)¦ x 00 C8 ¦&001C (2)¦ LinkPgm 00 ¦&001E (1)¦ x(Wnd) 00 ¦&001F (1)¦ x(Msg) FF FF ¦&0020 (2)¦ CRC 7E 00 ¦&0022 (2)¦ EOF | FEFE LINK 1-7,max 7 PgmID X X |

Recv:  7E 02 00 01 00 07 45 56 45 4E 00 09 55 FF FF 7E 00

| Command Bytes: | Description: |
| --- | --- |
| 7E 02 ¦&0000 (2)¦ Head of Frame 00 01 ¦&0002 (2)¦ Screen ID 00 07 ¦&0004 (2)¦ Data Length 45 56 45 4E ¦&0006 (4)¦ Cmd EVEN 00 ¦&000A (1)¦ Return Flag 09 ¦&000B (1)¦ Sub Cmd ID 55 ¦&000C (1)¦ Flag of Event0x55 FF FF ¦&000D (2)¦ CRC 7E 00 ¦&000F (2)¦ EOF | 00 Correct,0xaa Error 0x55 is In Event Driven Status |

## 2.10 Will be come: Print Image

Command Function:

Print a bitmap in user defined area. Print Mode: OR Mode, AND Mode, XOR Mode, and OVER Mode.

# 3. Feature and Escape String

## 3.1 How to display existed image in text string

In a text string, the code start of 0x80, and the second byte is in the range of 0x10-0x1f are index code of font matrix library.

First Code Byte,  0x80 – 0xff

Seconds Code Byte,  0x10 – 0x1f

e.g.

Insert First Font Matrix, 0x80, 0x10

Insert Second Font Matrix,0x80, 0x11

…

Insert 16th Font Matrix, 0x80,0x1f

Insert 17th Font Matrix, 0x81,0x10

Insert 18th Font Matrix, 0x81,0x11

…

## 3.2 Normal Escape String

| Escape String | Function | Others |
| --- | --- | --- |
| [FL000] | Font Matrix Library ID | [FL000] [FL0C8] |
| [CH8] | Height Pixels of Lines | [CH8] [CH12] [CH16] |
| [CR] | Return |  |
| [CXC] | Center [CXC], Left[CXL], Right[CXR] | Align |
| [CV] | FLASH Memory Variable |  |
| [Cv] | RAM Memory Variable |  |
| [DY] | Year |  |
| [DL] | Month |  |
| [DD] | Date |  |
| [DH] | Hour |  |
| [DM] | Minute |  |
| [DS] | Second |  |
| [DW] | Week |  |
| [DN] | Chinese Lunar |  |

## 3.3 Color Code and Color Escape String

| Escape String | Color Code BYTE |  | Font Color Name: English |
| --- | --- | --- | --- |
| [FC00] [FC01] [FC02] [FC03] [FC07] [FC08] [FC09] [FC0D] | 0x00 0x01 0x02 0x03 0x04 0x05 0x06 0x07 |  | 0=DEFAULT 1=RED 2=GREEN 3=YELLOW 4=BLUE 5=PINK 6=CYAN 7=WHITE |

* Notice: [FC00] – [FC0F] is max 16 colors, but Color Code is max 8 colors

## 3.4 Action Code

| Escape String | Action Code BYTE | Action Name: English |  |
| --- | --- | --- | --- |
| None | 0x00 0x01 0x02 0x03 0x04 0x05 0x06 0x07 0x08 0x09 0x0a 0x0b 0x0c 0x0d 0x0e 0x0f 0x10 | AUTO HOLD ROTATE ROTATE RIGHT SHUTTER SNOW SPARKLE TRIANGLE WIDE ROLL LEFT ROLL RIGHT ROLL UP ROLL DOWN WIPE LEFT WIPE RIGHT WIPE DOWN WIPE UP |  |

## 3.5 Asian/English Font ID Escape String

* Means it maybe not exist

| Escape String | Font Code BYTE[2] | Font Name | Charset |
| --- | --- | --- | --- |
| [FT100] [FT101] [FT102] [FT108] [FT110] [FT111] [FT112] [FT118] [FT120] [FT121] | 0x0100 0x0101 0x0102 0x0108 0x0110 0x0111 0x0112 0x0118 0x0120 0x0121 | 7x5 Sans Serif 7x6 Sans Serif 7x7 Sans Serif 12x8 EFont 16x8 Normal 16x8 Thin 16x8 Thick 16x8 Digit * 14x12 PMincho * 16x8 Mincho | Western |
| [FT200] [FT210] [FT300] [FT310] | 0x0200 0x0210 0x0300 0x0310 | SongTi 12 SongTi 16 * Tranditional 12 * Tranditional 16 | Aisian |
| [FT500] | 0x0500 | Korean Code Page |  |

## 3.6 Stay Time Code

| Escape String | Code (BYTE) | Stay Seconds | Description |
| --- | --- | --- | --- |
| None | 0x00 0x01 0x02 0x03 0x04 0x05 0x06 0x07 0x08 0x09 0x0a 0x0b 0x0c 0x0d 0x0e | 0.5s(Default) 0s 0.02s 0.04s 0.10s 0.25s 0.50s 0.75s 1s 2s 3s 5s 7.5s 10s 15s |  |

=== End of File ===